MRI Scanning Checklist – U of I MRI Research Facility

- Complete the MRI Research Protocol form and MRI Screening Sheet.

- Obtain subject IRB consent.

- Ensure subject places belongings and any metal objects into locker.

- Have subject walk-through metal detector.

- Familiarize subject with the scanner. This includes showing them the table, the surrounding magnet, and describing the sounds they may hear during the scan.

- Position the subject. This includes giving them earplugs, headphones (if needed), putting a clean sheet on the table, placing the knee pad under them, offering a blanket, and making sure they have the emergency squeeze ball.

- Landmark the subject using the laser system.

- Move the subject to the magnet isocenter.

- Adjust light & airflow for subject comfort as needed.

- Check on subject comfort prior to leaving magnet room and closing door.

- Check intercom communication system & adjust sound level if needed.

- Register the patient in the scanner.

- Select protocol and run the localizer.

- Run sequences, checking on the subject between sequences or as necessary.

- Enter the subject into the MRI logbook at console.

- Remove subject from the magnet.

- Archive data to XNAT and DCPACS and mark the task complete in the MRI logbook.

- Clean up magnet room and console area.

- Document any problems and issues with the scanner in the MRI logbook.

- Notify Miranda or Autumn of any technical issues that arise.

[Miranda cell: 319-415-5357, Autumn cell: 319-231-1221]

- Leave Research Protocol Form/MRI Screening Sheet and the signed IRB Informed Consent document on Miranda or Autumn’s desk so they can upload them to the scheduler.

- Turn lights off in the scan room and turn key to OFF on RF door.

Safe Scanning Program | The University of Iowa MRI Research Facility | 2025

Lock scanner room door key in lock box on wall.